# White Mountain Apache

Fire & Rescue Department

Adopted 09/02/08

Occupational Safety and Health

SOP 404.01 Operating on the Fire Ground

- Purpose

  - This policy was promulgated to:

    - Prevent accidents, injuries and deaths that might result from an unsafe act while employees are operating at an emergency incident.

    - Define the minimum personnel requirements for the safe conduct of emergency scene operations.

- Scope

  - This policy applies to all operational employees (trainee, recruits, firefighters, emergency medical technicians and fire officers), engaged in structural firefighting.

- Policy

  - Risk Management Guidelines

    - No member shall engage in firefighting activity unless they have met the requirements set forth in NFPA 1001.

    - No member shall commence or perform any firefighting or rescue function or evolution that is not within the established safety criteria of the department.

    - Each member shall practice as a “Safe Person” for their own safety, as well as to minimize risk to others. Towards that goal, all employees are expected to operate under the following risk management profiles:

      - Activities that present a significant risk to the safety of a firefighter shall be limited to situations where there is a potential to save endangered lives.

      - It is unacceptable to risk the safety of a firefighter when there is no chance of saving lives or property.

      - In situations where the risk to a member is excessive, activities shall be limited to defensive operations.

    - When an inexperienced member is working at an incident, direct supervision by an
experienced officer or other experienced personnel shall be provided.

    - Personnel operating in hazardous areas at emergency incidents shall operate in teams of two or more.   Team employees operating in hazardous areas shall be in constant communication with each other through visual, auditory, or physical means or through the use of a safety guide rope so as to coordinate their activities. Team employees shall remain in close proximity to each other to provide assistance in case of emergency.

    - An interior firefighting effort at a working structural fire shall not take place until a minimum of four firefighters are present.

      - When only four firefighters are present, two firefighters shall work as a team in the hazardous area and two firefighters shall be available for entry into the hazardous area if assistance or rescue is required.

      - A working fire is defined as a fire that requires the use of a 1-3/4" or larger attack line and the use of an SCBA.

    - The stand-by firefighters shall be responsible for maintaining a constant awareness of the number and identity of the employees operating in the hazardous area, their location and function, and their time of entry. The standby employees shall remain in radio, visual, voice, or signal line communication with the team.

    - One standby member shall be permitted to perform other duties outside of the hazardous area, such as serving as an apparatus operator or incident commander, provided that constant communication is maintained between the stand-by member and the employees of the team in the hazardous area.

      - The assignment of personnel to other duties shall not be permitted if the abandonment of their assignment would jeopardize the safety and health of any firefighter working at the incident.

      - The assignment of personnel to other duties shall not be permitted if their assignment inhibits the ability to assist or perform a rescue.

      - Standby firefighters shall have full protective clothing and SCBA available to them as defined by SOP 403.01, Protective Clothing and Equipment.

    - Exception: Rescue operations may be undertaken prior to assembly of four firefighters if there is imminent life-threatening situation and immediate action could prevent the loss of life or serious injury. No exception shall be permitted when there is no chance of saving lives.

    - When a second team is assigned to or begins operating in the hazardous area, the incident commander shall designate a least one rapid intervention team (RIT) to stand by in the event that a rescue becomes necessary.

      - The RIT shall consist of at least two firefighters. The team shall have full protective clothing and SCBA available to them as defined by SOP 403.01, Protective Clothing and Equipment.

      - During the initial stages of an incident, the RIT may be used to perform other functions provided that the team is immediately available to perform a rescue if so required.

      - As an incident grows in complexity, the number of RITs shall be increased proportionately and shall be solely dedicated to this responsibility.

    - At least one ambulance shall stand by during all working incidents and be available to treat injuries and to provide transport if necessary.

    - Firefighters operating from aerial devices shall be secured to the device by an approved safety harness.

    - Apparatus shall be used as a shield against oncoming traffic wherever possible.

  - Emergency Coordination

    - RITs shall be provided with portable radios and shall monitor the fire ground frequency.

    - The term MAYDAY shall be used by anyone on the scene who becomes aware of or is involved in a life-threatening situation.

    - The term EMERGENCY TRAFFIC shall be used by anyone on the scene who needs to communicate an urgent message.

    - Three (3) loud blasts from an apparatus air horn shall be used in addition to using the radio to order an immediate evacuation of a structure.

    - During Mayday or Emergency Traffic conditions, all other incident radio traffic shall immediately stop.

- Responsibilities

  - It shall be the responsibility of each member to fully comply with the provisions of this standard.

  - Officers shall be responsible for keeping their crews together and for ensuring that they do not expose their crews to unnecessary risks.

  - The incident commander shall be responsible for ordering sufficient resources to ensure that all emergency incident functions are performed in a safe manner.