# White Mountain Apache

Fire & Rescue Department

Adopted 06/07/2010

Rules and Regulations,

SOP 502.04 Rope Rescue Log

- Purpose

  - The purpose of this policy is to establish guidelines concerning the documentation of the ropes used and its condition after usage.

- Scope

  - For the general safety of all personnel involved, the condition of the rope will be documented in a rope log and this rope log will be filled out after every use.

- Policy

  - Each rope bag will have its own log card. The log card shall consist of the following:

    - Manufacturer

    - Diameter

    - Design

    - Tensile strength

    - Date of Purchase

    - Critical Data

  - The log card should have enough space to allow rope technicians to note each time the rope was used and for what activity.

  - The rope log may be stored on or in the rope bag or in a designated place on the apparatus it is stored in.

  - Specific entries must be made whenever the rope was subjected to abuse that could affect its performance or safety.

  - The rope log card shall be completed after every use, including training, actual incidents and inspection activities. Every user must follow this discipline; otherwise, the rope history is incomplete.

  - Rope can be inspected two ways, by look and by feel.

    - When visually inspecting the rope, look for the following:

      - Discoloration (could indicated chemical damage)

      - Glossy marks (heat fusion damage)

      - Exposed core fibers (damage to the sheath)

      - Lack of uniformity in diameter and size (broken sheath bundles)

      - Inconsistency in texture and stiffness (core damage)

    - For tactile inspection, run the rope slowly through your bare hands and feel for the following:

      - Stiffened fibers

      - Obvious changes in diameter

      - Soft or hollow spots

      - Contamination with dirt and grit

  - It is essential that entries for each rope be made every time ropes are returned to storage after use.

- Roles & Responsibilities

  - The on duty Captain will be responsible for the timely completion of the inspections and inventorying of the equipment. The Captain can appoint a designated person to carry out the inspection and inventory.

  - All personnel shall assume personal responsibility for the care and maintenance of the rope rescue equipment. The date, use, and condition will be noted in the rope log.