# White Mountain Apache

Fire & Rescue Department

Adopted 07/07/09

Minimum Company Standards

SOP 800.03 MCS – Minimum Equipment List (Engine)

- Purpose

  - Standardization of apparatus, equipment and supplies is critical to the successful deployment of resources for the control and mitigation of fire, rescue and medical incidents, as well as the safety of the crew.

- Scope

  - This policy applies to all pumper (non-tender) apparatus utilized for initial fire attack.

- Policy

  - Our objective is to ensure that all fire apparatus are equipped and supplied the same in accordance with NFPA 1901 2003 Edition regardless of the service area.  This is to ensure tactical and strategic goals and objectives can be accomplished.

  - All fire apparatus shall meet the following minimum requirements for equipment inventory.

    - Hose

      - 1200’ of Supply Hose

        - Minimum 600’ of 5” LDH w/ 5” storz to 4 ½” female adapter to facilitate a hydrant connection

        - Minimum 600’ of 3” hose set up in reverse lay.

      - 200’ of 2 ½” or 3” hose pre-connected to rear 2 ½” discharge, equipped with 2 ½” smoothbore nozzle.

      - 20’ Hard/Flex suction hose

      - 25’ LDH Soft suction hose (i.e. pony section) w/ 5” storz to 4 ½” female adapter to facilitate a hydrant connection.

      - 25’ of 3” hose

      - 400’ total of 1 ¾” pre-connected hoselines.

        - Pre-connects to be color coded to facilitate identification.

      - 200’ spare feet of 1 ¾” hose.

      - 200’ of 1” forestry hose

      - 300’ of ¾” forestry hose

    - Appliances

      - Pump intake piston relief valve

      - Deck gun with portable base and stack tips.

      - If non-detachable, one portable monitor

      - Two (2) 2 ½” to two (2) 1 ½” gated wyes

      - 2 ½” siamese to 5” storz (optional, if available)

      - 2 ½” siamese to 6” female (pump intake)

      - Three (3) 2 ½” double female coupling

      - Three (3) 2 ½” double male coupling

      - One (1) 2 ½” male to 5” storz adapter

      - One (1) 2 ½” female to 5” storz adapter

      - 2 ½” female to 1 ½” male reducer adapater

      - 2 ½” gate valve

      - Two (2) 1 ½” double male coupling

      - Two (2) 1 ½” double female coupling

      - Assortment of Forestry adapters (e.g. Green List)

    - CAFS Equipped Pumpers

      - Three (3) 15/16” CAFS Nozzles

      - Three (3) Fog Nozzle Attachments

      - ½” Smoothbore Reducer Nozzle

      - 2 ½” Smoothbore Nozzle (on 2 ½” hose)

      - 2 ½” Fog Nozzle (optional, if available)

      - Medium Expansion Nozzle (i.e. Mad Dog)

      - Set Stack Tip

      - Piercing Nozzle (1st out apparatus only)

    - Non-CAFS Equipped Pumpers

      - Three (3) 1 ½” Fog Nozzles

      - 2 ½” Fog Nozzle (optional, if available)

      - 2 ½” Smoothbore Nozzle (on 2 ½” hose)

      - Set Stack Tips

      - Foam Inductor w/ Pick-up Tube

      - Foam Nozzle Attachment (1st out apparatus only)

    - Ladders

      - 2 or 3 section extension ladder

      - Roof ladder

      - Folding or attic ladder

    - Hand-tools

      - Three (3) Spanner/Hydrant Wrenches Set

        - Six (6) spanner wrenches

        - Three (3) hydrant wrenches

      - Two (2) storz wrenches

      - Two axes (one pickhead and one flathead)

      - One (1) hooligan/haligan bar

      - Pike poles

        - One (1) >8’ pike pole

        - One (1) <8’ pike pole

      - Trash hook

      - 8 lb. sledge hammer

      - Rubber mallet

      - Bolt cutter

      - Pipe wrench

    - SCBA

      - Minimum of Four (4) SCBA

      - Minimum Four (4) spare SCBA bottles

      - RIC connection or RIC kit

    - Powered Equipment

      - PPV fan

      - Saws (circular and chain saw)

      - Circular saw blades for metal and wood.

      - Portable power generator

      - Portable scene lights

      - Extension cord/reel

      - Pair Saw Chaps

    - Miscellaneous

      - Two (2) salvage covers (canvas or poly) 12 x 14

      - Hose clamp

      - Two (2) Wheel chocks

      - Binoculars (1st and 2nd out apparatus)

      - Minimum Four (4) safety vest

      - One (1) Hose strap

      - Two (2) shovels (Flat and Pointed)

      - Scoop shovel

      - 100’ search/guide rope w/ two (2) prusiks and two (2) carabineers

      - Fire extinguishers

        - Water extinguisher

        - Multi-purpose (>80B:C)

      - Drinking water

      - Fuel

        - <5 gallon gas can w/ unleaded fuel

        - 1 gallon gas can w/ fuel mix

      - Set mechanic tools

      - Tow chains

      - GPS receiver (1st & 2nd out apparatus)

      - WD40

      - Box flares

      - Road Triangles

      - Two (2) flashlights (mounted)

      - Combustible gas indicator (1st out apparatus only)

      - Two (2)Spray bottles

        - Soap solution

        - Bleach solution (10% concentration)

      - Reserved

    - References

      - Latest edition of ERG book

      - NIOSH pocket guide to hazardous materials

      - Resource listing

      - Map book

      - Clip board w/ pens

      - Scratch paper/field notes

    - Medical (Required on 1st out apparatus)

      - Oxygen delivery

        - Oxygen tank w/ regulator

        - Three (3) adult non-rebreather

        - Four (4) adult nasal cannula

        - Two (2) child non-rebreather

        - Two (2) child nasal cannula

        - Adult, Child and Infant BVM

      - AED w/ Spare Pads

      - Portable suction

      - Assortment OPA and NPAs

      - Bulb syringe

      - Tongue depressor

      - Bleeding Control

      - 20 Sterile 4 x 4’s

      - Two (2) occlusive dressing

      - Six (6) kling (small and large)

      - Two (2) ace bandages

      - Two (2) trauma dressing

      - Four (4) abdomen pads

      - Burn sheet

      - Band-Aids

      - OB Kit

      - Immobilization

      - Two (2) long backboards

      - Two (2) backboard straps

      - Assortment of cervical collars

      - Two (2) head rolls/blocks

      - Roll duct tape

      - KED or short board

      - Assortment extremity splints

      - Traction splint

      - Two (2) ice & heat packs

      - Adult and child BP cuff

      - Stethoscope

      - Pulse oximeter

      - Blood glutose meter

      - Scissors

      - Ammonia inhalants

      - Two (2) activated charcoal

      - Four (4) oral glucose tubes

      - Three (3) size HEPA mask

      - Bio hazard “red” bags

      - Saline and sterile water

      - Children’s chewable aspirin

    - Extrication

      - Hydraulic tools – spreader, cutter and ram(s) w/ power plant and hoses

      - Lift bags with controller, regulator and hoses

      - Assortment of cribbing

        - Buttress (ARS struts)

        - Step chocks

        - 4 x 4’s and/or 2 x 4’s

        - Wedges

        - Base cribbing

      - Jack (bottle)

      - Crash kit (i.e. window punch, batter terminal cutters, hack saws, etc.)

    - Foam

      - 15 [spare] gallons Class A foam

  - Reserved